Access Control System

Smart Card Reader

AC-ACQR201-CR

QR Code and Mifare Card Reader

Key Features:

- 3 color LED light with buzzer indicator

- Intergrated QR code and RFID technology

- Scratch-resistant acrylic front panel (4H hardness)

- Optional colors, can be customized for the end-user panel

- design

- Encrypted QR Code data, provides QR code generator tool

- and secondary development API

- Reverse polarity protection, all inputs and outputs have DC

- 12V and electrostatic protection

- Piezoelectric ceramic buzzer, ensuring consistent sound

- quality for mass-produced products

- Supports resetting reader parameters via QR code and

- configuration card

- Standard Wiegand interface, optional TCP/IP network

- interface and RS485 interface

Specification

| Model | AC-ACQR201-CR |
| --- | --- |
| Dimensions | 92mm*46.6mm*92mm |
| Rated voltage | DC 7V ~ 15V |
| Rated current | 100mA(max) |
| Operating temperature | -30°C ~ 70°C |
| Operating Humidity | 10~90% |
| Reading Distance | >5cm |
| Signal Transmission Distance | 150m |
| Data Interface | Wiegand & RS485 |