Access Control System

Door Controller

AC-ACNC4

4-Door Network Controller

Key Features:

- Rich product architecture, with up to 8 card reader interfaces implemented flexibly through RS485 serial ports, enabling bidirectional access control for 4 doors

- It supports storage of complete access control for up to 250,000 cardholders

- There is support for recording up to 100,000 offline card swipes and door opening events.

- All activities are reported to the host system, including supervised inputs/alarms with 255 priority levels

- The system can operate offline for all access control decisions and event logging even without communication with the host access control software application

- It provides 4 Wiegand interfaces and 4 serial interfaces for connecting card readers, allowing for greater flexibility to accommodate various types of card readers

- The system implements dual-door monitoring, dual REX switches, and customizable 4-channel inputs for events such as AC failure, battery failure, and tamper alarms

| Specification |  |
| --- | --- |
| Model | AC-ACNC4 |
| Processor | Industrial-grade NXP i.MX6UL chip with Arm® Cortex®-A7 core, running at a speed of 696 MHz, 32-bit processor |
| Installation | Installed on any wall with four screws or mounted on a standard 35mm DIN rail |
| Dimensions | 150mm (width) x 130mm (height) x 60mm (depth) |
| Weight | 395 grams (net weight), 541 grams (gross weight) |
| Casing Material | Material Made of polycarbonate |
| Operating environment | Indoor, or inside a cabinet that complies with NEMA-4 standards |
| Operating environment temperature | 0°C to 50°C |
| Operating environment humidity | Operating environment humidity 5% to 85% (non-condensing) |
| Communication Ports | 10/100 Mbps Ethernet interface, CAN bus |
| Operating VDC/Current (max) | Power input: 12VDC/2.5A; Lock output: 12VDC/4A |
| Wiegand/RS485 Reader Interface | Wiegand/RS485 Reader Interface Each 12VDC output supports a maximum current of 0.5A |
| SIA standard Wiegand/serial card reader interface | Four ports available for each interface |
| Lock input and output VDC | 12VDC |
| Lock output current per port (max) | 1A |
| Certificates | RoHS, EMC, CE, FCC |