Access Control System

Door Expansion Module

AC-ACEMO16

16-port Output Expansion Module

Key Features:

- Customizable 4-channel inputs, such as AC failure, battery failure, and tamper alarm

- Each input point can be programmed for abnormal conditions (can use NO or NC alarm devices)

- 16 latching relays with contact rating of 1A @ 30VDC

- Connected to AC-ACNC series controllers via CAN bus

- Receives and processes real-time commands from AC-ACNC series controllers

- Reports all activities to AC-ACNC series controllers

- Allows for complex input/output linking when used with AC -ACNC series controllers and AC-ACEMI16

- Encased in polycarbonate packaging to prevent component damage

- All connections and indicator lights are clearly labeled with silk-screened text

| Specification |  |
| --- | --- |
| Model | AC-ACEMO16 |
| Processor | Industrial-grade ST chip with Arm® Cortex®-M3 core, running at a speed of 72 MHz, 32-bit processor |
| Installation | Installed on any wall with four screws or mounted on a standard 35mm DIN rail |
| Dimensions | 150mm (width) x 130mm (height) x 60mm (depth) |
| Weight | 313 grams (net weight), 521 grams (gross weight) |
| Casing Material | Made of polycarbonate |
| Operating environment | Indoor, or inside a cabinet that complies with NEMA-4 standards |
| Operating environment temperature | 0°C to 50°C |
| Operating environment humidity | 5% to 95% (non-condensing) |
| Communication Ports | CAN bus |
| Operating VDC/Current (max) | Power input: 12VDC/1.5A; Lock output: 12VDC/2A |
| Input interfaces | 16 normally open/normally closed relays, with contact rating of 1A @ 30VDC |
| Certificates | RoHS, EMC, CE, FCC |