Access Control System

Door Controller

AC-ACNC32

32-Door Network Main Controller

Key Features:

- Rich product architecture allows for flexible implementation of access control for up to 32 doors byconnecting AC-ACEM2 or AC-ACEM4 interface expansion panels through the CAN bus

- Through the CAN bus, it can achieve fire linkage for multiple devices

- Supports recording of up to 200,000 offline card swipes and door opening events

- Supports storage of complete access control and configuration database for up to 500,000 cardholders

- Can be connected to control I/O expansion panels to meet corresponding functional requirements

- Reports all activities to the host system; reports supervised inputs/alarms with 255 priority levels

- Provides comprehensive offline operation functionality, including executing all access control decisions and event logging, even without communication with the host access control software application

- Provides 4 Wiegand interfaces and 4 serial interfaces for connecting card readers, allowing for greater flexibility to support various types of card readers

- Allows for customization of 4 -channel inputs, such as AC failure, battery failure, and tamper alarms

| Specification |  |
| --- | --- |
| Model | AC-ACNC32 |
| Processor | Industrial-grade NXP i.MX6UL chip with Arm® Cortex®-A7 core, running at a speed of 696 MHz, 32-bit processor, 256M/512M memory and 4G/8G storage |
| Installation | Installed on any wall with four screws or mounted on a standard 35mm DIN rail |
| Dimensions | 110mm (width) x 130mm (height) x 60mm (depth) |
| Weight | 256 grams (net weight), 472 grams (gross weight) |
| Casing Material | Made of polycarbonate |
| Operating environment | Indoor, or inside a cabinet that complies with NEMA-4 standards |
| Operating environment temperature | -15°C to 60°C |
| Operating environment humidity | 5% to 85% (non-condensing) |
| Communication Ports | 10/100 Mbps Ethernet interface, CAN bus |
| Operating VDC/Current (max) | Power input: 9-15VDC/0.5A |
| Certificates | RoHS, EMC, CE, FCC |