Access Control System

Door Expansion Module

AC-ACEMI16

16-port Input Expansion Module

Key Features:

- Supports managed or unmanaged alarm circuits

- Each input point can be programmed for abnormal conditions

- Supports 4-state mode input with adjustable programmable design

- Connected to AC-ACNC series network bus controllers via CAN bus

- Receives and processes real-time commands from AC-ACNC series network bus controllers

- Reports all activities to AC-ACNC series network bus controllers

- Allows for complex input/output linking when used with AC -ACNC series network bus

- controllers and AC-ACEMO16

- Encased in polycarbonate packaging to prevent component damage

- All connections and indicator lights are clearly labeled with silk-screened text

- 2 relay outputs, each interface with both NO and NC interfaces

| Specification |  |
| --- | --- |
| Model | AC-ACEMI16 |
| Processor | Industrial-grade ST chip with Arm® Cortex®-M3 core, running at a speed of 72 MHz, 32-bit processor |
| Installation | Installed on any wall with four screws or mounted on a standard 35mm DIN rail |
| Dimensions | 110mm (width) x 130mm (height) x 60mm (depth) |
| Weight | 290 grams (net weight), 504 grams (gross weight) |
| Casing Material | Made of polycarbonate |
| Operating environment | Indoor, or inside a cabinet that complies with NEMA-4 standards |
| Operating environment temperature | 0°C to 50°C |
| Operating environment humidity | 5% to 95% (non-condensing) |
| Communication Ports | CAN bus |
| Operating VDC/Current (max) | Power input: 12VDC/0.5A |
| Input interfaces | 16 input loops available, supporting NO or NC configurations, with support for custom programming |
| Certificates | RoHS, EMC, CE, FCC |