Access Control System

Smart Card Reader

AC-ACR6180

Contactless Smart Card Reader

Key Features:

- The card reader offers flexible installation options, allowing it to be securely matched and fixed with select electrical switch installation boxes

- The housing is made of high-strength PC+ABS and tempered glass materials, providing resistance against strong impacts

- Adopts dynamic encryption using 3DES and SM1 (China National Standard 1, Commercial Standard 1)

- An advanced key management system is implemented to reduce the risk of data theft or duplication

- Easily interface with most existing Wiegand protocol access control panels

- Configurable output options include Wiegand26, Wiegand32, Wiegand34, Wiegand35, Wiegand36,Wiegand37, and Wiegand44

- Supports ISO/IEC 14443 TYPE A/B and ISO/IEC 15693 communication protocols

- Bluetooth 4.0 is supported (optional) for smart phone Bluetooth door opening functionality

- QR code recognition is supported (optional) for smart phone QR code door opening functionality

- Online detection alarms are supported (requires connection with ACNC series controllers)

- Local tamper alarms and remote triggering are supported (requires ACNC series controllers)

| Specification |  |
| --- | --- |
| Model | AC-ACR6180 |
| Installation | Installed on any wall with four screws, compatible with EU/APAC-60mm/GB |
| Dimensions | 86mm (width) x 100mm (height) x 11mm (depth) (Installed) |
| Input VDC | 12VDC |
| Standby operating current | 60mA |
| Trigger operating current | 90mA Max |
| Sensing distance | 0-60mm (0”-2.4”) |
| Operating environment temperature | -15°C -55°C（ 5°F -131°F） |
| Operating environment humidity | 5% to 90% (non-condensing) |
| Operating environment | Indoor |
| Color | Black |
| IP code | IP55 |