Access Control System

Door Expansion Module

AC-ACEM2

2-Door Expansion Module

Key Features:

- Report Management Input - Connected to AC-ACNC32 via CAN bus

- Receives and processes real-time commands from AC-ACNC32

- Reports all activities to AC-ACNC32, reports supervised inputs/warnings

- Encased in polycarbonate packaging to prevent component damage

- All connections and indicators clearly labeled with silkscreen text

- Offline access control decisions made based on device codes

- Customizable with 4 inputs, such as AC faults, battery faults, and tamper alarms

| Specification |  |
| --- | --- |
| Model | AC-ACEM2 |
| Processor | Industrial-grade ST chip with Arm® Cortex®-M3 core, running at a speed of 72 MHz, 32-bit processor |
| Installation | Installed on any wall with four screws or mounted on a standard 35mm DIN rail |
| Dimensions | 110mm (width) x 130mm (height) x 60mm (depth) |
| Weight | 290 grams (net weight), 504 grams (gross weight) |
| Casing Material | Made of polycarbonate |
| Operating environment | Indoor, or inside a cabinet that complies with NEMA-4 standards |
| Operating environment temperature | 0°C to 50°C |
| Operating environment humidity | 5% to 95% (non-condensing) |
| Communication Ports | CAN bus |
| Operating Voltage/Current (max) | Power input: 12VDC/1.5A; Lock output: 12VDC/2A |
| Wiegand/RS485 Reader Interface | Each 12VDC output supports a maximum current of 0.5A |
| SIA standard Wiegand/serial card reader interface | Two ports available for each interface |
| Lock input and output VDC | 12VDC |
| Signal Transmission Distance | 150m |
| Data Interface | Wiegand & RS485 |