Access Control System

Door Controller

AC-ACNC2P

Enhanced 2-Door Network Controller

Key Features:

- Supports bus network configuration, effectively ensuring the security and reliability of bus communication

- Rich product architecture allows flexible implementation of access control for up to 32 doors by connecting AC-MT2 or AC-MT4 interface expansion panels through the CAN bus

- Supports recording of up to 200,000 offline card swipes and door opening events

- Supports storage of complete access control for up to 500,000 cardholders

- Can be connected to servers and other devices on TCP/IP networks

- Can be connected to input and output expansion panels to meet corresponding functional requirements

- Receives and processes real-time commands from the host software application

- Reports all activities to the host system; reports supervised inputs/alarms with 255 priority levels

| Specification |  |
| --- | --- |
| Model | AC-ACNC2P |
| Processor | Industrial-grade NXP i.MX6UL chip with Arm® Cortex®-A7 core, running at a speed of 696 MHz, 32-bit processor; 256M memory; 4G storage |
| Installation | Installed on any wall with four screws or mounted on a standard 35mm DIN rail |
| Dimensions | 110mm (width) x 130mm (height) x 60mm (depth) |
| Weight | 281 grams (net weight), 497 grams (gross weight) |
| Casing Material | Made of polycarbonate |
| Operating environment | Indoor, or inside a cabinet that complies with NEMA-4 standards |
| Operating environment temperature | -15°C to 60°C |
| Operating environment humidity | 5% to 85% (non-condensing) |
| Communication Ports | 10/100 Mbps Ethernet interface, CAN bus |
| Operating VDC/Current (max) | Power input: 9-15VDC/1.5A; |
| Wiegand/RS485 Reader Interface | Each 12VDC output supports a maximum current of 0.5A |
| SIA standard Wiegand/serial card reader interface | Two ports available for each interface |
| Lock input and output VDC | 12VDC/2.4A |
| Lock output current per port (max) | 1.2A |
| Certificates | RoHS, EMC, CE, FCC |
| Model | AC-ACNC2P |
| Processor | Industrial-grade NXP i.MX6UL chip with Arm® Cortex®-A7 core, running at a speed of 696 MHz, 32-bit processor; 256M memory; 4G storage |
| Installation | Installed on any wall with four screws or mounted on a standard 35mm DIN rail |